LabView Driver Reference for 5007 Dual Frequency Synthesizer

10/27/2011

## List of Functions

Read / Write Register

Read / Write Reference Oscillator Frequency

Read / Write Synthesizer Label

Read / Write Frequency Range

Read / Write Frequency Settings

Read / Write Control Bits

Write Flash

Set Synthesizer Level

Set Synthesizer Output

## Function Description

## InitConnection

InitConnection initializes a connection to the Synthesizer board and returns a reference to the connection.

## CloseConnection

CloseConnection closes a connection to the Synthesizer board.

## Read Register

Read Register reads all the register data from the Synthesizer board for the selected synthesizer and returns the values.

Parameters

- Reference to the connection to the Synthesizer

- Synthesizer - This selects which a Synthesizer to access, Synthesizer A or Synthesizer B.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference

- Register 0 to 5 – Register data returned by the Synthesizer Board

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Write Register

Write Register writes all the register data for the Synthesizer board for the selected synthesizer.

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Register 0 to 5 – Register data to write to the Synthesizer.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Success? – True if the Synthesizer board returned a acknowledgement, if it failed, this is set to False.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Read Reference Oscillator Frequency

Read Reference Frequency reads the Synthesizer’s reference frequency from the board and returns the value.

Parameters

- Reference to the connection to the Synthesizer.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Reference Frequency – The reference frequency returned by the synthesizer board.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Write Reference Oscillator Frequency

Write reference frequency sets the reference frequency

Parameters

- Reference to the connection to the Synthesizer

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Success? – True if the Synthesizer board returned a acknowledgement, if it failed, this is set to False.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Read Synthesizer Label

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Label – Returns the selected Synthesizer’s label.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Write Synthesizer Label

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Label – The new label to be sent to the selected synthesizer.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Success? – True if the Synthesizer board returned an acknowledgement, if it failed, this is set to False.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Read Frequency Range

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Minimum Frequency Range – The minimum frequency range returned by the synthesizer.

- Maximum Frequency Range – The maximum frequency range returned by the synthesizer.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Write Frequency Range

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Minimum Frequency Range – The minimum frequency range to be sent to the synthesizer.

- Maximum Frequency Range – The maximum frequency range to be sent to the synthesizer.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Success? – True if the Synthesizer board returned an acknowledgement, if it failed, this is set to False.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Read Frequency Setting

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Write Frequency Setting

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Frequency – Sets the desired output frequency

- Frequency Increment – Sets the frequency increment

- Frequency Offset – Sets the frequency Offset

- Phase Detector Frequency – Sets the phase detector frequency

- Reference Doubler – True will use the reference doubler

- Reference Divide by 2 – True will use the divide by 2 setting

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Success? – True if the Synthesizer board returned a acknowledgement, if it failed, this is set to False.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Read Control Bits

Parameters

- Reference to the connection to the Synthesizer.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Synthesizer A Locked – Returns true if the Synthesizer A is locked, false if it is not locked

- Synthesizer B Locked – Returns true if the Synthesizer A is locked, false if it is not locked

- Reference External Oscillator Selected – Returns true if the external reference is selected and false if the internal reference is used.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Write Control Bits

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Success? – True if the Synthesizer board returned an acknowledgement, if it failed, this is set to False.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Write Flash

Write Flash writes the Synthesizer’s RAM buffer to the flash.

Parameters

- Reference to the connection to the Synthesizer.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Success? – True if the Synthesizer board returned an acknowledgement, if it failed, this is set to False.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Set Synthesizer Level

This function is used to set a synthesizer’s wave level.

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Level – The options are -2 to 7, the higher the number, the higher the level.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Success? – True if the Synthesizer board returned an acknowledgement, if it failed, this is set to False.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example

## Set Synthesizer Output

This function is used to turn OFF and ON a synthesizer’s output.

Parameters

- Reference to the connection to the Synthesizer.

- Synthesizer - This selects which Synthesizer to access, Synthesizer A or Synthesizer B.

- Output On – False will set the synthesizer’s output to OFF and True will set the output to ON.

- Error In - The current Error status can be passed in, if there is an error, the function will not do anything and pass the error to error out.

Output

- Connection reference – The Synthesizer connection reference.

- Success? – True if the Synthesizer board returned an acknowledgement, if it failed, this is set to False.

- Error Out – The error status returned by the function, if an error occurred during execution, the error information can be read from this reference.

Example